### 6.2.5 Driveways

A.	Residential.

- Single-car driveways in all residential districts shall be a minimum of ten feet in width.

- Double-car driveways in all residential districts shall be a minimum of eighteen feet in width.

- Driveways may not be located within 30 feet of a street intersection. Driveways must be a minimum distance of five feet from a side or rear lot line unless an application is made for a joint driveway with an adjacent premise.

- Residential driveways and curb cuts must be approved through a Land Use Permit prior to construction.

- Driveways to single-family dwellings shall be located in the greater side yard setback.

B.	Commercial and Industrial

- Driveways shall be designed to accommodate the type and volume of vehicular traffic using the site and located in a manner that is compatible with uses located adjacent to and across from the site.

- Single-lane (truck access) driveways shall be a minimum of fifteen feet in width and no wider than twenty feet.

- Two-lane (truck access) driveways shall be a minimum of thirty feet in width and no wider than forty feet.

- Apron width may extend beyond the driveway for smooth entry/exit.

- The Planning Commission, depending on the commercial or industrial land use and its circulation requirements, may adjust these requirements based on its site plan.

- Properties having access to a state highway will require an access (driveway approach)  permit from the Michigan Department of Transportation (MDOT).